# 信息论复习题满分复习文档 / Full-Mark Review Notes for Information Theory

范围说明 / Scope: 本文只围绕 复习题-2026 2.docx 中出现的知识点、题型和标准答案整理。考试是英文题目，所以每个知识点都给出英文答题表达。

统一约定 / Convention: 若无特别说明，所有对数均为以 2 为底，单位为 bit 或 bit/symbol.

## 0. 考前总策略 / Exam Strategy

中文：

- 选择填空题要背“概念边界”：自信息随概率增大而减小；平均互信息非负；单个互信息可以为负；率失真端点；信道容量范围；极限熵不大于各阶平均符号熵。

- 大题基本都是同一套路：先列概率分布，再代入熵、互信息、容量或编码公式。

- 计算题必须写过程：概率、公式、代入、结果、单位。只写结果容易丢过程分。

- 线性分组码所有运算都在 GF(2) 上，即加法为模 2 加法。

- Huffman/Fano 编码的 0/1 可以整体互换，但平均码长和效率必须一致。若题目给了“unique Huffman coding”，优先按复习题里的码字口径写。

English:

- For short questions, memorize the conceptual boundaries: self-information decreases as probability increases; average mutual information is non-negative; pointwise mutual information may be negative; the endpoints of the rate-distortion function; the range of channel capacity; the limiting entropy is not greater than finite-order average symbol entropies.

- For calculation problems, always list the probability distribution first, then apply the formula.

- Show probability, formula, substitution, numerical result, and unit.

- For linear block codes, all additions are modulo-2 operations over GF(2).

- For Huffman/Fano coding, exchanging all 0 and 1 labels gives an equivalent code. Use the code pattern in the review sheet when possible.

# 1. 必背概念题 / Must-Memorize Concepts

## 1.1 Shannon 通信系统 / Shannon Communication System

中文答案：

Shannon 通信系统的基本组成包括：信源、信道、信宿。

English answer:

The components of the Shannon communication system include source, channel, and destination.

常见填空：

source, channel, destination

## 1.2 自信息 / Self-Information

公式 / Formula:

I(x_i) = -log P(x_i) = log(1 / P(x_i))

中文：

某事件概率越大，不确定性越小，自信息越小；概率越小，不确定性越大，自信息越大。

English:

As the probability of an event increases, its self-information decreases. A less probable event carries more information.

必背句：

As the probability of a certain event increases, its self-information decreases.

特殊值 / Special cases:

P(x)=1  -> I(x)=0
P(x) smaller -> I(x) larger

## 1.3 熵 / Entropy

公式 / Formula:

H(X) = - sum_i p(x_i) log p(x_i)
     = sum_i p(x_i) I(x_i)

中文：

熵是平均自信息，表示信源平均不确定性。

English:

Entropy is the average self-information of a source, measuring the average uncertainty of the source.

常用数值 / Useful values:

log 2 = 1
log 3 = 1.5850
log 4 = 2

-0.5 log 0.5 = 0.5
-0.4 log 0.4 = 0.5288
-0.3 log 0.3 = 0.5211
-0.2 log 0.2 = 0.4644
-0.1 log 0.1 = 0.3322

## 1.4 单个互信息与平均互信息 / Mutual Information vs Average Mutual Information

单个互信息 / Pointwise mutual information:

I(x_i; y_j) = log [ p(x_i, y_j) / (p(x_i)p(y_j)) ]

平均互信息 / Average mutual information:

I(X;Y) = sum_i sum_j p(x_i,y_j) log [ p(x_i,y_j) / (p(x_i)p(y_j)) ]
       = H(X) + H(Y) - H(X,Y)
       = H(X) - H(X|Y)
       = H(Y) - H(Y|X)

中文：

平均互信息一定非负；单个互信息在某些条件下可以为负。

English:

The average mutual information is always non-negative, while pointwise mutual information can be negative under certain conditions.

选择题高频答案：

The mutual information can be negative in certain condition.

注意：

这里题目里的 mutual information 指单个事件互信息，不是 average mutual information.

## 1.5 联合熵与条件熵 / Joint Entropy and Conditional Entropy

联合熵 / Joint entropy:

H(X,Y) = - sum_i sum_j p(x_i,y_j) log p(x_i,y_j)

条件熵 / Conditional entropy:

H(Y|X) = - sum_i sum_j p(x_i,y_j) log p(y_j|x_i)
H(X|Y) = - sum_i sum_j p(x_i,y_j) log p(x_i|y_j)

常用关系 / Useful identities:

H(X,Y) = H(X) + H(Y|X)
H(X,Y) = H(Y) + H(X|Y)
I(X;Y) = H(X) + H(Y) - H(X,Y)

## 1.6 N 次扩展信源熵 / Entropy of N-th Extension Source

对离散无记忆信源 / For a discrete memoryless source:

H(X^N) = N H(X)

若题目问单符号信源 entropy 与 N extended source entropy 的关系，答案一般选：

H(X^N) = N H(X)

English sentence:

For a discrete memoryless source, the entropy of the N-th extension source is N times the entropy of the original single-symbol source.

## 1.7 极限熵 / Limiting Entropy or Ultimate Entropy

定义 / Definition:

H_infinity = lim_{N->infinity} H(X_N | X_1, X_2, ..., X_{N-1})

对平稳有记忆信源，平均符号熵随阶数增加不增：

H_infinity <= ... <= H_N/N <= ... <= H_2/2 <= H_1

复习题选择题答案：

The ultimate entropy must be smaller than any single entropy.

更严谨表达：

The limiting entropy is not greater than any finite-order average symbol entropy.

## 1.8 率失真函数端点 / Endpoints of Rate-Distortion Function

必背：

R(D_min) = H(X)
R(D_max) = 0

中文：

允许失真最小时，相当于无失真编码，所需信息率为 H(X)。允许失真达到最大可容许失真时，不需要传输信息，R(Dmax)=0。

English:

At the minimum distortion, the rate-distortion function equals the source entropy. At the maximum allowable distortion, the required rate becomes zero.

## 1.9 信道容量范围 / Range of Channel Capacity

复习题填空口径：

0 <= C <= min{H(X), H(Y)}

English:

For the channel capacity from the source X to the destination Y, the range of C is from 0 to min{H(X), H(Y)}.

更一般地：

C = max_{p(x)} I(X;Y)

# 2. DMS + 噪声信道计算题 / DMS and Noisy Channel Calculation

## 2.1 标准步骤 / Standard Procedure

题型：

Given a DMS X and a noisy channel P(Y|X), calculate self-information, H(X), H(Y), H(X,Y), and I(X;Y).

中文步骤：

- 根据 P(X) 求自信息 I(a_i)。

- 求 H(X)。

- 由全概率公式求 P(Y)。

- 求 H(Y)。

- 求联合概率 P(X,Y)=P(X)P(Y|X)。

- 求 H(X,Y)。

- 求 I(X;Y)=H(X)+H(Y)-H(X,Y)。

English template:

First, we calculate the marginal distribution of Y by the law of total probability.
Then the joint distribution is obtained by p(x_i,y_j)=p(x_i)p(y_j|x_i).
Finally, the average mutual information is
I(X;Y)=H(X)+H(Y)-H(X,Y).

## 2.2 复习题原型 / Review Problem Prototype

已知 / Given:

P(a1)=0.5, P(a2)=0.5

P(Y|X) =
        b1    b2
a1     0.6   0.4
a2     0.2   0.8

### a) Self-information

I(a1) = -log 0.5 = 1 bit
I(a2) = -log 0.5 = 1 bit

English answer:

The self-information values are I(a1)=1 bit and I(a2)=1 bit.

### b) Source entropy H(X)

H(X) = 0.5*1 + 0.5*1 = 1 bit/symbol

### c) Marginal distribution and entropy of Y

P(b1) = 0.5*0.6 + 0.5*0.2 = 0.4
P(b2) = 0.5*0.4 + 0.5*0.8 = 0.6

H(Y) = -0.4 log 0.4 - 0.6 log 0.6
     = 0.97095 bit/symbol
     ≈ 0.971 bit/symbol

### d) Joint distribution and joint entropy

P(a1,b1) = 0.5*0.6 = 0.3
P(a1,b2) = 0.5*0.4 = 0.2
P(a2,b1) = 0.5*0.2 = 0.1
P(a2,b2) = 0.5*0.8 = 0.4

H(X,Y)
= -0.3log0.3 -0.2log0.2 -0.1log0.1 -0.4log0.4
= 0.5211 + 0.4644 + 0.3322 + 0.5288
= 1.8465 bit/symbol

### e) Average mutual information

I(X;Y) = H(X) + H(Y) - H(X,Y)
       = 1 + 0.97095 - 1.8465
       = 0.1245 bit/symbol
       ≈ 0.125 bit/symbol

考试注意 / Exam note:

复习题答案中最后一行写成了 I(X,Y)=H(X)+H(Y)-I(X,Y)，这是笔误。正确公式是：

I(X;Y)=H(X)+H(Y)-H(X,Y)

English correction:

The correct formula is I(X;Y)=H(X)+H(Y)-H(X,Y).

# 3. 二阶 Markov 信源 / Binary 2-Step Markov Source

## 3.1 状态定义 / State Definition

题型：

A binary 2-step Markov source has symbol set {0,1}. The transition probability of the next symbol depends on the previous two symbols.

中文：

二阶 Markov 信源的状态不是单个符号，而是长度为 2 的符号组：

S1=00, S2=01, S3=10, S4=11

English:

For a second-order binary Markov source, each state is a two-symbol state: 00, 01, 10, and 11.

## 3.2 复习题原型 / Review Problem Prototype

已知 / Given:

P(0|00)=P(1|11)=0.6
P(1|00)=P(0|11)=0.4
P(0|01)=P(0|10)=P(1|01)=P(1|10)=0.5

### a) 状态转移图 / State Transition Graph

写法：

00 -> 00 with probability 0.6
00 -> 01 with probability 0.4

01 -> 10 with probability 0.5
01 -> 11 with probability 0.5

10 -> 00 with probability 0.5
10 -> 01 with probability 0.5

11 -> 10 with probability 0.4
11 -> 11 with probability 0.6

English sentence:

The next state is obtained by shifting left and appending the newly generated symbol.

### b) 状态转移矩阵 / State Transition Matrix

按状态顺序 (00, 01, 10, 11):

P =
[ 0.6  0.4  0    0   ]
[ 0    0    0.5  0.5 ]
[ 0.5  0.5  0    0   ]
[ 0    0    0.4  0.6 ]

### c) 平稳分布 / Stationary Distribution

设 / Let:

w = (w1, w2, w3, w4)

满足 / It satisfies:

wP = w
w1 + w2 + w3 + w4 = 1

结果 / Result:

w = (5/18, 2/9, 2/9, 5/18)

English answer:

The stationary state distribution is (5/18, 2/9, 2/9, 5/18) for states (00,01,10,11).

### d) 极限符号分布 / Limiting Symbol Distribution

P(0) = P(0|00)w1 + P(0|01)w2 + P(0|10)w3 + P(0|11)w4
     = 0.6*(5/18) + 0.5*(2/9) + 0.5*(2/9) + 0.4*(5/18)
     = 1/2

P(1) = 1 - P(0) = 1/2

English answer:

The limiting distribution of symbols 0 and 1 is (1/2, 1/2).

## 3.3 Markov 链平稳分布通用模板 / General Template for Stationary Distribution

English template:

Let pi be the stationary distribution. It must satisfy pi = pi P and sum_i pi_i = 1.
Solving these equations gives the stationary distribution.

中文模板：

设平稳分布为 pi，列方程 pi = pi P 和归一化条件 sum pi_i=1，解方程即可。

## 3.4 Markov 极限熵 / Limiting Entropy of a Markov Chain

对一阶 Markov 链：

H_infinity = sum_i pi_i H(P_i)

其中 pi_i 是平稳状态概率，P_i 是第 i 行条件分布。

English:

The limiting entropy of a Markov chain is the weighted average of the entropy of each row of the transition matrix, with stationary probabilities as weights.

# 4. Jensen-Shannon Divergence 与 Markov 综合题

## 4.1 JS 散度公式 / Jensen-Shannon Divergence

给两个分布 p 和 q:

m = (p + q) / 2
JS(p,q) = 1/2 KL(p||m) + 1/2 KL(q||m)

KL 散度：

KL(p||q) = sum_i p_i log(p_i / q_i)

等价熵公式：

JS(p,q) = H((p+q)/2) - [H(p)+H(q)]/2

English:

The Jensen-Shannon divergence is a symmetric and bounded version of KL divergence.

## 4.2 复习题原型 / Review Problem Prototype

已知两个条件分布 / Given:

p = (0.7, 0.3)
q = (0.4, 0.6)

### a) JS divergence

复习题答案：

JS(p,q) = 0.0462 bit/symbol

English answer:

The Jensen-Shannon divergence of the two conditional distributions is 0.0462 bit/symbol.

### b) Stationary distribution

若转移矩阵为 / If:

P =
[ 0.7  0.3 ]
[ 0.4  0.6 ]

设 / Let:

pi = (pi_F, pi_D)

解：

pi = pi P
pi_F + pi_D = 1

结果：

pi = (4/7, 3/7)

### c) Limiting entropy

H_infinity = (4/7)H(0.7,0.3) + (3/7)H(0.4,0.6)
           = 0.9199 bit/symbol

English answer:

The limiting entropy of the Markov chain is 0.9199 bit/symbol.

# 5. 离散信道与信道容量 / Discrete Channel and Channel Capacity

## 5.1 基本公式 / Basic Formula

信道容量定义：

C = max_{p(x)} I(X;Y)

English:

Channel capacity is the maximum average mutual information over all possible input distributions.

## 5.2 对称信道 / Symmetric Channel

如果一个离散无记忆信道是对称信道，并且每行概率分布相同排列，输出符号数为 m，则：

C = log m - H(row)

复习题 A 型：

row = (4/5, 1/10, 1/10)
m = 3

C = log 3 - H(4/5, 1/10, 1/10)
  = 1.5850 - 0.9219
  = 0.6631 bit/symbol

English answer:

Channel A is a symmetric channel, and its capacity is 0.6631 bit/symbol.

## 5.3 准对称信道 / Quasi-Symmetric Channel

中文：

准对称信道可以分解为若干个对称子信道。其容量取决于每行熵、子信道列和以及输入符号数。

English:

A quasi-symmetric channel can be decomposed into several symmetric subchannels. Its capacity is computed from the row entropy and the column sums of the subchannels.

复习题 B 型答案：

C = 0.126 bit/symbol

答题句：

Channel B is a quasi-symmetric channel. By decomposing it into symmetric subchannels and applying the quasi-symmetric channel capacity formula, the capacity is 0.126 bit/symbol.

## 5.4 医疗 AI 准对称信道题 / Medical AI Quasi-Symmetric Channel

题型：

Medical AI system classifies chest X-ray images into Healthy, Pneumonia, and Uncertain. The channel is quasi-symmetric.

复习题给出的容量计算：

C = log 2 - (0.9log0.9 + 0.1log0.2) - H(0.7,0.2,0.1)
  = 1 + 0.369 - 1.1568
  = 0.2122 bit/symbol

标准答案：

C = 0.2122 bit/symbol

英文答题：

The channel is quasi-symmetric. Substituting the row entropy and the subchannel terms into the capacity formula gives C = 0.2122 bit/symbol.

### 0.2 与 0.7 对调时 / If 0.2 and 0.7 are swapped

中文：

容量保持不变。原因是准对称信道的子对称信道行和、列和结构不变，只是概率位置发生交换。

English answer:

The channel capacity remains unchanged, because swapping 0.2 and 0.7 only changes the positions of probabilities. The row sums and column sums of the quasi-symmetric subchannels remain unchanged.

# 6. 率失真函数 / Rate-Distortion Function

## 6.1 必背公式 / Must-Memorize Formula

R(D_min) = H(X)
R(D_max) = 0

中文解释：

失真最小意味着无失真恢复，需要完整保留信源熵。失真最大时，可以完全不传信息，信息率为 0。

English explanation:

At minimum distortion, lossless reconstruction is required, so the rate equals the source entropy. At maximum allowable distortion, no information needs to be transmitted, so the rate is zero.

选择填空常见答案：

R(Dmin)=H(X), R(Dmax)=0

# 7. Shannon-Fano / Fenno Coding 与 Huffman Coding

复习题里写的是 Fenno coding，通常指 Shannon-Fano coding。

## 7.1 平均码长 / Average Code Length

公式：

K_bar = sum_i p_i l_i

English:

The average code length is the weighted sum of codeword lengths.

## 7.2 编码效率 / Coding Efficiency

公式：

eta = H(X) / K_bar

百分比：

eta = H(X) / K_bar * 100%

English:

The coding efficiency is the ratio of source entropy to average code length.

## 7.3 Shannon-Fano 编码步骤 / Shannon-Fano Coding Procedure

中文步骤：

- 按概率从大到小排序。

- 将符号集合分成概率总和尽量接近的两组。

- 一组赋 0，另一组赋 1。

- 对每个子组继续二分，直到每个符号都有唯一码字。

English template:

Sort the symbols in descending order of probability. Split the set into two groups with probabilities as close as possible. Assign 0 to one group and 1 to the other group, and repeat recursively.

## 7.4 Huffman 编码步骤 / Huffman Coding Procedure

中文步骤：

- 选概率最小的两个符号合并。

- 新节点概率为两者之和。

- 重复合并直到只剩一个根节点。

- 从根到叶分配 0/1，得到码字。

English template:

At each step, combine the two symbols with the smallest probabilities. Repeat this operation until one root node remains. Assign binary labels to the branches to obtain the Huffman code.

## 7.5 复习题原型 1：五符号 DMS / Review Prototype 1: Five-Symbol DMS

概率分布：

a1: 0.4
a2: 0.2
a3: 0.2
a4: 0.1
a5: 0.1

### Shannon-Fano / Fenno code

复习题码字：

a1 -> 0
a2 -> 100
a3 -> 101
a4 -> 110
a5 -> 111

平均码长：

K_bar = 1*0.4 + 3*0.2 + 3*0.2 + 3*0.1 + 3*0.1
      = 2.2 bit/symbol

### Huffman code

复习题码字：

a1 -> 1
a2 -> 01
a3 -> 001
a4 -> 0000
a5 -> 0001

平均码长：

K_bar = 1*0.4 + 2*0.2 + 3*0.2 + 4*0.1 + 4*0.1
      = 2.2 bit/symbol

熵：

H(X) = -0.4log0.4 -2*0.2log0.2 -2*0.1log0.1
     = 2.122 bit/symbol

编码效率：

eta = 2.122 / 2.2 = 0.9645 = 96.45%

English answer:

Both coding methods have the same average code length, 2.2 bit/symbol, and the coding efficiency is about 96.45%.

## 7.6 复习题原型 2：传感器四符号 Huffman / Review Prototype 2: Four-Symbol Sensor Huffman

概率分布：

A: 0.4
B: 0.3
C: 0.2
D: 0.1

复习题 Huffman code:

A -> 1
B -> 01
C -> 000
D -> 001

平均码长：

L = 0.4*1 + 0.3*2 + 0.2*3 + 0.1*3
  = 1.9 bit/symbol

熵：

H(X) = -0.4log0.4 -0.3log0.3 -0.2log0.2 -0.1log0.1
     = 1.8464 bit/symbol

编码效率：

eta = H(X)/L * 100%
    = 1.8464/1.9 * 100%
    = 97.18%

English answer:

The average code length is 1.9 bit/symbol and the coding efficiency is 97.18%.

# 8. 线性分组码 / Linear Block Coding

## 8.1 基本概念 / Basic Concepts

线性分组码 (n,k):

k = number of message bits
n = number of codeword bits

编码：

x = uG

其中 u 是消息，G 是生成矩阵。

English:

For an (n,k) linear block code, the codeword is obtained by multiplying the message vector by the generator matrix over GF(2).

## 8.2 GF(2) 运算 / Operations over GF(2)

0+0=0
0+1=1
1+0=1
1+1=0

中文：

线性分组码里的加法不是普通整数加法，而是模 2 加法。

English:

All additions in linear block coding are modulo-2 additions.

## 8.3 监督矩阵 / Parity-Check Matrix

若生成矩阵是系统形式：

G = [ I_k | P ]

则监督矩阵：

H = [ P^T | I_{n-k} ]

满足：

G H^T = 0

English:

If the generator matrix is in systematic form G=[I_k|P], then the parity-check matrix is H=[P^T|I_{n-k}].

## 8.4 Hamming 距离与检错纠错 / Hamming Distance, Error Detection and Correction

最小 Hamming 距离：

d_min = minimum Hamming distance between any two distinct codewords

检错能力：

detect up to d_min - 1 errors

纠错能力：

correct up to floor((d_min - 1)/2) errors

English answer:

A code with minimum Hamming distance d_min can detect up to d_min-1 errors and correct up to floor((d_min-1)/2) errors.

## 8.5 Syndrome 译码 / Syndrome Decoding

收到码字 w:

S = w H^T

若 S=0，无错或错误未检出。

若 S 等于 H 的某一列，则该列对应位置发生单比特错误。

纠正：

corrected codeword = w + e

English template:

The syndrome is S=wH^T. Since the syndrome equals the corresponding column of H, the error pattern is identified. Therefore, the transmitted codeword is w+e.

## 8.6 复习题原型 1：(6,3) 线性分组码 / Review Prototype 1: (6,3) LBC

复习题中由码字可还原的生成矩阵：

G =
[1 0 0 1 1 0]
[0 1 0 0 1 1]
[0 0 1 1 0 1]

### a) Encode M=(100)

x = (100)G = (100110)

English answer:

The codeword for message M=(100) is (100110).

### b) Parity-check matrix H

这里：

P =
[1 1 0]
[0 1 1]
[1 0 1]

所以：

H = [P^T | I_3]

H =
[1 0 1 1 0 0]
[1 1 0 0 1 0]
[0 1 1 0 0 1]

### c) Received code w=(101111), error weight < 2

S = wH^T = (1 0 0)^T

S 等于 H 的第 4 列，所以错误图样：

e = (000100)

纠正码字：

a = w + e
  = 101111 + 000100
  = 101011

English answer:

The syndrome is (1,0,0)^T, which corresponds to an error in the fourth bit. Therefore, the transmitted LBC code is (101011).

### d) 全部码字 / All codewords

000 -> 000000
001 -> 001101
010 -> 010011
100 -> 100110
011 -> 011110
110 -> 110101
101 -> 101011
111 -> 111000

最小距离：

d_min = 3

纠错能力：

correct up to floor((3-1)/2)=1 error

检错能力：

detect up to 3-1=2 errors

## 8.7 复习题原型 2：另一道 (6,3) 线性分组码 / Review Prototype 2: Another (6,3) LBC

题目要求：

a) Encode u=[101]
b) Determine d_min
c) Explain error detection ability

复习题答案核心：

uG = [1 0 1 1 0 1]

即：

codeword = 101101

距离计算中出现：

d(G1,G2)=4
d(G1,G3)=4
d(G2,G3)=4
G1+G2=110110
G1+G3=101101
G2+G3=011011

复习题给出：

d_min = 2

按正确公式：

detect up to d_min - 1 = 1 error

注意：

复习题截图里有一处把 d_min=2 后的检错数写成 2 errors 的笔误；题目正文旁边也出现 =1 error，考试时建议按公式写：

A code with minimum distance d_min can detect up to d_min-1 errors.
Here d_min=2, so it can detect up to 1 error.

# 9. 天气-健康综合题 / Weather-Health Entropy and Mutual Information

## 9.1 题型 / Problem Type

题目：

12 examples of school students describing clothing, weather condition, and health states. Let X be weather condition {cold, comfy, cool} and Y be health state {healthy, sick}.

要求：

a) probability distributions of X and Y
b) H(X) and H(Y)
c) joint probability distribution and H(X,Y)
d) I(X;Y)

## 9.2 复习题标准答案 / Standard Answer

边缘分布 / Marginal distributions:

P_X(cold)  = 1/4
P_X(comfy) = 1/3
P_X(cool)  = 5/12

P_Y(healthy) = 2/3
P_Y(sick)    = 1/3

熵 / Entropies:

H(X) = 1.5546 bit/symbol
H(Y) = 0.9183 bit/symbol

联合分布 / Joint distribution:

cold    comfy   cool
healthy      1/6     1/4     1/4
sick         1/12    1/12    1/6

联合熵 / Joint entropy:

H(X,Y) = 2.4591 bit/symbol

互信息 / Mutual information:

I(X;Y) = H(X) + H(Y) - H(X,Y)
       = 1.5546 + 0.9183 - 2.4591
       = 0.0138 bit/symbol

English answer:

The average mutual information between weather condition and health state is 0.0138 bit/symbol, which indicates that the dependence between X and Y is very weak in this sample.

# 10. 选择题与填空题标准答案 / Standard Answers for Objective Questions

## 10.1 Single-Choice Questions

### Q1

题意：

For the entropy of a single-symbol discrete source, the entropy of the N extended source must satisfy …

答案：

B

应背关系：

H(X^N) = N H(X)

### Q2

答案：

B

原句：

The mutual information can be negative in certain condition.

解释：

这里指单个互信息或点互信息；average mutual information 不会为负。

### Q3

答案：

A

原句：

The ultimate entropy must be smaller than any single entropy.

更严谨写法：

The limiting entropy is not greater than any finite-order average symbol entropy.

## 10.2 Completion Questions

### a)

source, channel, destination

### b)

decrease

完整句：

As the probability of a certain event increases, its self-information would decrease.

### c)

R(Dmin)=H(X), R(Dmax)=0

### d)

0 and min{H(X),H(Y)}

完整句：

The range of C should be among 0 and min{H(X),H(Y)}.

# 11. 英文考试答题模板 / English Answer Templates

## 11.1 Entropy Calculation

The entropy of X is calculated as
H(X) = -sum_i p(x_i)log p(x_i).
Substituting the given probabilities, we obtain H(X)=... bit/symbol.

## 11.2 Joint Entropy Calculation

The joint probabilities are obtained by p(x_i,y_j)=p(x_i)p(y_j|x_i).
Then
H(X,Y) = -sum_i sum_j p(x_i,y_j)log p(x_i,y_j).
Therefore, H(X,Y)=... bit/symbol.

## 11.3 Mutual Information Calculation

The average mutual information is
I(X;Y)=H(X)+H(Y)-H(X,Y).
Substituting the calculated entropy values gives I(X;Y)=... bit/symbol.

## 11.4 Markov Stationary Distribution

Let pi be the stationary distribution. It satisfies pi=pi P and sum_i pi_i=1.
Solving these equations gives pi=(...).

## 11.5 Markov Limiting Entropy

The limiting entropy is the stationary weighted average of the conditional entropies:
H_infinity = sum_i pi_i H(P_i).

## 11.6 Symmetric Channel Capacity

For a symmetric channel, the capacity is
C = log m - H(row).
Substituting the row distribution gives C=... bit/symbol.

## 11.7 Quasi-Symmetric Channel Capacity

The channel is quasi-symmetric. It can be decomposed into symmetric subchannels.
Using the quasi-symmetric channel capacity formula, we obtain C=... bit/symbol.

## 11.8 Huffman Coding

We combine the two symbols with the smallest probabilities at each step.
The resulting Huffman code is ...
The average code length is L=sum_i p_i l_i=... bit/symbol.
The coding efficiency is eta=H(X)/L=...%.

## 11.9 Linear Block Coding

The codeword is obtained by x=uG over GF(2).
The syndrome is S=wH^T.
Since the syndrome corresponds to the ... column of H, the error is in the ... bit.
Therefore, the corrected codeword is ...

## 11.10 Error Detection Ability

A code with minimum Hamming distance d_min can detect up to d_min-1 errors and correct up to floor((d_min-1)/2) errors.

# 12. 最后一页速记 / One-Page Cheat Sheet

I(x) = -log p(x)

H(X) = -sum p(x)log p(x)

H(X,Y) = -sum sum p(x,y)log p(x,y)

H(Y|X) = -sum sum p(x,y)log p(y|x)

I(X;Y) = H(X)+H(Y)-H(X,Y)
       = H(X)-H(X|Y)
       = H(Y)-H(Y|X)

For DMS:
H(X^N)=N H(X)

Rate distortion:
R(Dmin)=H(X)
R(Dmax)=0

Channel capacity:
C=max I(X;Y)
0 <= C <= min{H(X),H(Y)}

Symmetric channel:
C=log m - H(row)

Stationary distribution:
pi=pi P, sum pi_i=1

Markov limiting entropy:
H_infinity=sum pi_i H(P_i)

JS divergence:
JS(p,q)=H((p+q)/2)-[H(p)+H(q)]/2

Average code length:
L=sum p_i l_i

Coding efficiency:
eta=H(X)/L

Linear block code:
x=uG
S=wH^T
G=[I|P] -> H=[P^T|I]

Minimum distance:
Detect d_min-1 errors
Correct floor((d_min-1)/2) errors

# 13. 复习题数值答案汇总 / Numerical Answers from the Review Sheet

Objective questions:
Single choice: B, B, A
Completion:
source / channel / destination
decrease
R(Dmin)=H(X), R(Dmax)=0
0, min{H(X),H(Y)}

DMS noisy channel:
I(a1)=1 bit
I(a2)=1 bit
H(X)=1 bit/symbol
P(Y)=(0.4,0.6)
H(Y)=0.971 bit/symbol
H(X,Y)=1.846 bit/symbol
I(X;Y)=0.125 bit/symbol

2-step Markov source:
State order: 00,01,10,11
Stationary distribution: (5/18,2/9,2/9,5/18)
Limiting symbol distribution: P(0)=1/2, P(1)=1/2

Channel capacity:
A: symmetric channel, C=0.6631 bit/symbol
B: quasi-symmetric channel, C=0.126 bit/symbol
Medical AI quasi-symmetric channel: C=0.2122 bit/symbol
Swapping 0.2 and 0.7: capacity remains unchanged

Five-symbol coding:
P=(0.4,0.2,0.2,0.1,0.1)
Fenno average length: 2.2 bit/symbol
Huffman average length: 2.2 bit/symbol
H(X)=2.122 bit/symbol
Efficiency=96.45%

Sensor Huffman:
P(A,B,C,D)=(0.4,0.3,0.2,0.1)
Code: A=1, B=01, C=000, D=001
L=1.9 bit/symbol
H(X)=1.8464 bit/symbol
Efficiency=97.18%

Linear block code:
M=(100) -> codeword 100110
Received 101111 -> corrected codeword 101011
d_min=3 for that code
Can correct 1 error and detect 2 errors

Another LBC:
u=[101] -> 101101
d_min=2
Can detect 1 error

Weather-health:
P_X=(1/4,1/3,5/12)
P_Y=(2/3,1/3)
H(X)=1.5546
H(Y)=0.9183
H(X,Y)=2.4591
I(X;Y)=0.0138

Study-habit Markov + JS:
JS((0.7,0.3),(0.4,0.6))=0.0462
Stationary distribution=(4/7,3/7)
H_infinity=0.9199 bit/symbol